TECHNICAL SPECIFICATION SHEET

SHADEPORT SYSTEMS

STRUCTURAL STEEL FRAMEWORK — CANTILEVERS

Anchor Poles: 152mm / 165mm round tubing (3mm wall thickness — heavy-duty)

Main Frame: 50mm / 57mm round tubing (2mm wall thickness)

STRUCTURAL STEEL FRAMEWORK — FOUR POST

Anchor Poles: 76mm / 101mm round tubing (3mm wall thickness — heavy-duty)

Main Frame: 50mm round tubing (2mm wall thickness)

FOUNDATIONS & ANCHORING

600mm – 800mm deep pole planting

Secured with ready-mix concrete

WIND & LOAD PERFORMANCE

Balanced cantilever load distribution

Designed for high wind conditions (120–140 km/h when correctly installed)

SHADE NETTING

Knittex Z25 UV-stabilised shade netting

High durability and tear resistance

CABLE TENSIONING SYSTEM (CERTIFIED)

5mm 6x19 Fibre Core Galvanised Steel Cable

Breaking Load: 13.6 kN (≈ 1,385 kg)

Tested Breaking Load: 14.3 kN

Secured with Crosby clamps

STITCHING & THREAD — COATS DABOND AWF

Bonded, twisted continuous filament polyester thread

PFC-free anti-wick finish blocks water migrating through stitched seams — a prerequisite for genuinely water-repellent seams, not just weatherproof fabric

Dual-level lubrication protects against needle heat and keeps high-speed sewing smooth

“Z”-twist construction, purpose-built for single-needle high-speed industrial sewing

Excellent bleach, mildew and rot resistance for long-term outdoor exposure

Two tex weights specified by seam load: Tex 80 (5,700 cN breaking strength) for standard seams, Tex 135 (9,310 cN breaking strength) for heavy-duty seams

INSTALLATION STANDARD

Precision alignment of cantilever arms

Correct structural positioning

Professional tensioning and finishing

SYSTEM SUMMARY

Heavy-duty steel framework

Deep concrete foundations

High-strength certified cable system

Coats-certified stitching for water- and weather-resistant seams

Built for long-term durability and performance

Prepared By: Minette Liebenberg

Director

Shade Solutions by FacilitiesCo

sales@facilitiesco.com  ·  010 015 0532 / 083 378 5122

www.facilitiesco.com  ·  @facilitiesco